Instructions on using the LPJavaCN30.htm test application to print to PB20

Connect CN30 to desktop using ActiveSync

Copy the 5 files over to the CN30 on the root of the handheld. Must be the Root.

InkSig.bmp 	- sample signature bitmap

linePtr.dll 	- Line printer activeX control

LPJavaCN30.htm – test html

Regsvrce.exe	- used to register activeX controls

Wince.ini	- used by lineptr.dll to set up printer and com port

Tap regsvrce.exe and type in lineptr.dll. Tap OK

Tap regsvrce.exe again to make sure lineptr.dll was registered

Turn Bluetooth on and discover PB20

Tap Start -> Settings -> System -> Wireless Printing at bottom of list

Tap Device Discovery and press Acquire Printer

Select the correct printer and tap OK

Current Wirless Printer screen should show the correct printer. Tap OK

Goto LPJavaCN30.htm and tap on button 1 to print to PB20